PROPOSED INFRASTRUCTURE
SOLUTION

2-Node Hyper-V Failover Cluster • Shared SAN • Redundant Network • DR Site • Veeam Backup

| Project | Digital Malawi Acceleration Project – Central Printing Facility |
| --- | --- |
| Reference | RFP No. MW-PPPC-546386-GO-RFB |
| Solution Scope | Primary site infrastructure, DR infrastructure, network, storage, backup and recovery |
| Document Status | High-Level Technical Solution / Tender Design Basis |
| Date | 26 September 2026 |

Design principle: no single infrastructure component should prevent operation of critical production services where redundancy is technically and commercially justified.

1. Executive Summary

This document presents a high-level infrastructure solution for the Central Printing Facility, based on the architectural, availability, security, backup and disaster-recovery requirements contained in Section VII of the RFP. The proposed design uses a two-node Microsoft Hyper-V failover cluster at the Primary Site with shared dual-controller SAN storage, redundant Layer-3 core switching, segmented VLANs, a protected DMZ, a dedicated backup platform and a secondary Disaster Recovery (DR) environment.

The design is intended to provide high availability for server workloads at the production site while also providing site-level resilience through replication to the DR site. Veeam Backup & Replication is proposed as the central data-protection platform for Hyper-V backup, application-aware recovery, VM replication, off-site backup copy, recovery verification and immutable backup storage.

Important sizing note: the RFP defines architectural, availability and performance outcomes but does not prescribe exact server CPU/RAM, SAN capacity or switch models. Quantities and capacities in this document are therefore preliminary design assumptions and must be finalized after the application vendor provides the VM, database, IOPS, capacity, growth and retention requirements.

1.1 Key Design Outcomes

Primary Site: two-node Hyper-V failover cluster with shared enterprise SAN.

DR Site: two-node Hyper-V failover cluster with independent shared SAN for critical workload recovery.

Redundant core switching at both sites to remove the core switch as a single point of failure.

Dedicated logical networks for management, cluster, storage, applications, database, machine control, users, backup and DMZ.

Veeam-based backup and replication with an immutable backup repository and off-site copy.

Encrypted inter-site replication over MPLS/SD-WAN or secure VPN, subject to available WAN service.

Operational procedures for backup, restore, planned failover, emergency failover and DR testing.

1.2 RFP Requirements Addressed

| RFP Requirement | Proposed Response |
| --- | --- |
| Secure, scalable, redundant enterprise-grade hardware | Redundant compute, dual-controller SAN, dual core switches, multipath storage and dual power paths. |
| Active-passive / active-active clustering and automated failover | Windows Server Failover Clustering with Hyper-V clustered workloads. |
| Primary and secondary DR environments | Independent Primary and DR virtualization/storage stacks. |
| Backup replication and off-site backup | Veeam backup copy / replication to DR repository. |
| Immutable backup storage | Veeam hardened/immutable repository or immutable object storage. |
| Point-in-time and granular recovery | Application-aware Veeam backup and supported item/file/VM restore workflows. |
| VLAN segmentation and firewall isolation | Dedicated security zones/VLANs with NGFW policy enforcement. |
| DR synchronization | VM/storage/data replication according to workload criticality and supported application design. |
| RTO/RPO definition and DR testing | Service tiers, runbooks, scheduled restore testing and annual/semi-annual DR drills. |

2. Proposed High-Level Architecture

The following detailed architecture separates production, DR and backup functions while preserving redundant paths between compute, network and storage layers. Application, database, API/integration, reverse proxy, monitoring, management tools and the Veeam Backup Server are virtual machines hosted by the Hyper-V clusters. Backup repositories are deliberately kept on separate physical infrastructure so that backup data does not share the same production storage failure domain.

Figure 1 - Detailed Primary Site, DR Site, VM Placement and Veeam Backup Architecture

2.1 Logical Traffic Flow

External NRIS/WAN traffic enters through the perimeter security layer and is allowed only to approved integration or DMZ services.

Application, database and machine-control networks remain segmented from user and external zones.

Each Hyper-V node connects redundantly to both core switches and to both SAN controllers.

Cluster heartbeat/live migration, storage, management and production traffic are logically separated.

Critical VMs are protected locally by failover clustering and protected against site failure by replication to the DR site.

Backup traffic uses a dedicated backup VLAN and repository rather than relying only on production storage snapshots.

2.2 Availability Domains

| Failure Scenario | Protection Mechanism | Expected Result |
| --- | --- | --- |
| Single Hyper-V node failure | WSFC + clustered VMs | VMs restart on surviving node; exact service time depends on guest/application startup. |
| Single core switch/link failure | Dual switches + redundant NIC paths | Traffic continues over surviving path. |
| Single SAN controller/path failure | Dual-controller SAN + MPIO | Storage remains accessible through alternate controller/path. |
| Production VM corruption | Veeam restore / instant recovery where supported | Recover VM, volume, file or application data from protected restore point. |
| Ransomware affecting production | Immutable backup copy separated from production credentials | Clean restore point remains protected for configured immutability period. |
| Primary site outage | DR VM/data replication + documented failover | Critical services recovered at DR according to service tier and dependency order. |

3. Primary Site Infrastructure

3.1 Hyper-V Failover Cluster

The production virtualization platform is proposed as a two-node Windows Server Failover Cluster (WSFC) running Hyper-V. Both nodes should be from the same OEM/server family, use compatible firmware and drivers, and be validated for the selected Windows Server release.

VM Placement Principle

The following services are virtualized on the two-node Hyper-V cluster: Domain Controllers/DNS, application servers, database servers, API/integration services, reverse proxy/web services, monitoring server, management tools server, Veeam Backup Server and other application VMs. The reverse proxy and API/integration VMs are logically connected to the DMZ/External Integration zone through VLAN and firewall policy even though they physically run on the cluster hosts.

The physical backup repository and immutable backup storage are not part of the Hyper-V cluster and do not reside on the production SAN. This separation preserves a distinct backup failure and security domain.

| Component | Preliminary Specification | Design Rationale |
| --- | --- | --- |
| Cluster nodes | 2 × enterprise 2U rack servers | Two nodes provide local high availability and permit maintenance of one node while services run on the other. |
| CPU | 2 sockets/server; preliminary 16–24 cores per CPU | Final sizing must be based on application/DB vendor requirements and licensing. |
| Memory | 256 GB RAM/server minimum design allowance; expandable to 512 GB+ | Provides headroom for failover; surviving node must carry critical workloads. |
| Boot | 2 × enterprise SSD in RAID 1 | Separates host OS from VM data. |
| Data/cluster NIC | Minimum 4 × 10/25GbE per host, or equivalent converged design | Redundant production, cluster/live migration, management and backup paths. |
| Storage connectivity | Dual-path 10/25Gb iSCSI or FC | No single storage path; use MPIO. |
| OOB management | Dedicated iLO/iDRAC/XClarity or equivalent | Remote hardware management independent of host OS. |
| Power | Dual hot-plug PSUs to separate PDUs/UPS feeds | Removes single PSU/power-feed dependency. |

3.2 Shared SAN Storage

A dedicated dual-controller enterprise SAN is proposed for clustered VM storage. The SAN should be sized from measured or vendor-provided capacity and IOPS requirements rather than from raw TB alone.

Dual active/active or active/standby controllers with non-disruptive controller failover.

Enterprise SSD / all-flash preferred for database and high-volume production workloads.

RAID-6 or vendor-equivalent protected pool, subject to performance validation.

Multipath connectivity from each Hyper-V node to both storage controllers.

Thin provisioning, snapshots and monitoring, while treating snapshots as operational aids rather than backups.

Encryption at rest where supported and required by the security design.

Initial design allowance: approximately 10–15 TB usable, subject to formal sizing.

Capacity reserve for growth, snapshots, maintenance operations and performance stability.

3.3 Primary Network

Two managed Layer-3 core switches are recommended. A single main switch would create a single point of failure and would weaken the RFP's redundancy objective. The switches should support VLANs, LACP/MLAG or vendor-equivalent multi-chassis resiliency, ACLs, QoS, SNMP/telemetry, 10/25Gb server uplinks and redundant power.

| VLAN | Purpose | Typical Members |
| --- | --- | --- |
| 10 | Infrastructure Management | Hyper-V management, OOB management gateways, infrastructure tools |
| 20 | Hyper-V Cluster / Live Migration | Cluster heartbeat and live migration |
| 30 | Storage / iSCSI | Hyper-V hosts and SAN interfaces only |
| 40 | Application | Application and workflow servers |
| 50 | Database | Database servers / DB cluster |
| 60 | Machine Control / Personalization | Industrial control and personalization systems |
| 70 | Management & Monitoring | Monitoring, administration, SIEM integrations |
| 80 | Users / Operators | Authorized operator and administrative workstations |
| 90 | Backup | Veeam server, proxies and repositories |
| 100 | DMZ / External Integration | Reverse proxy, API/integration gateways |

3.4 Edge Access Switch

For the currently identified small user/operator requirement, one managed 24-port PoE+ access switch is a practical baseline. It provides spare capacity for workstations, printers, IP phones, access points or management devices. If user access is operationally critical, a second access switch or stack should be included to remove the access-layer single point of failure.

4. Disaster Recovery Site

The DR site is designed as an independent recovery environment rather than an extension of the same failure domain. It should have independent power, network/security connectivity, compute, storage and backup capacity sufficient to recover the defined critical services.

| DR Component | Proposed Baseline | Notes |
| --- | --- | --- |
| DR Hyper-V cluster | 2 × enterprise servers | Prefer same or compatible architecture as production; target Hyper-V version equal to or newer than source for Veeam replication. |
| DR SAN | 1 × dual-controller SAN | Sized for replicated critical VMs plus recovery working space and growth. |
| DR core switches | 2 × redundant L3 switches | Independent resilient switching. |
| DR firewall | 1 HA pair preferred / minimum appropriately resilient NGFW design | Terminates secure site connectivity and enforces DR segmentation. |
| DR backup repository | 1 physical system | Independent physical off-site repository at DR for backup copies and recovery |
| WAN | MPLS/SD-WAN or encrypted VPN | Bandwidth sized from change rate, replication interval and backup-copy window. |

4.1 DR Recovery Tiers

| Tier | Example Workloads | Target Design | Comment |
| --- | --- | --- | --- |
| Tier 1 – Critical | Database, core application, integration required for production | Near-real-time / ≤5 min replication target where technically supported | Must be validated against application consistency and WAN capacity. |
| Tier 2 – Important | Management, monitoring, supporting application services | Frequent replication / backup copy | Recover after Tier 1 dependencies. |
| Tier 3 – Non-critical | Reporting, test, utility services | Backup-based recovery | Can tolerate longer restoration window. |

4.2 Failover Sequence

Declare disaster and freeze non-essential changes.

Confirm integrity and freshness of DR replicas/restore points.

Bring up identity, DNS and required infrastructure dependencies.

Recover database services and verify application consistency.

Recover application/integration services.

Recover management/monitoring and remaining services by priority.

Validate user access, external integration and production workflows.

Record recovery times, data point achieved and exceptions for the DR report.

5. Backup and Recovery Solution – Veeam

5.1  Veeam Backup & Replication

Veeam Backup & Replication is an appropriate solution for the proposed Microsoft Hyper-V environment. Current Veeam documentation lists support for Windows Server Hyper-V 2025, 2022, 2019 and 2016. Veeam also supports Hyper-V VM replication and immutable backup repositories. This makes it suitable for addressing the RFP requirements for centralized backup management, off-site replication, ransomware-resilient immutable backups and recovery workflows.

Product/version compatibility must be re-validated against the exact Veeam build, Windows Server build, storage firmware and application versions at implementation time.

5.2 Recommended Backup Architecture

The backup design follows a 3-2-1-1-0 style protection principle: multiple copies of data, on different storage/failure domains, at least one off-site copy, at least one immutable/offline-style copy, and routine verification. The Veeam Backup Server is deployed as a VM on the production Hyper-V cluster, while the primary backup repository is a separate physical server/storage system. An immutable repository is implemented on separate hardened physical infrastructure or a supported immutable target, and an independent repository is maintained at the DR site.

| Layer | Function | Proposed Technology |
| --- | --- | --- |
| Production data | Live VM and application data | Hyper-V cluster + shared SAN |
| Primary backup | Fast operational restore | Veeam Backup & Replication to dedicated backup repository |
| Immutable copy | Ransomware-resistant restore points | Veeam hardened Linux repository or supported immutable object/appliance repository |
| Off-site copy | Site-loss protection | Backup copy to DR repository over encrypted WAN |
| VM replication | Fast DR for selected critical VMs | Veeam Hyper-V replication to DR Hyper-V cluster |
| Recovery validation | Prove recoverability | Scheduled restore tests / Veeam verification capabilities as applicable |

5.3 Proposed Backup Schedule

| Protection | Frequency | Retention – Initial Proposal | Purpose |
| --- | --- | --- | --- |
| Critical VM incremental backup | Daily; increase frequency where RPO requires | 30 daily restore points | Operational recovery |
| Weekly full / synthetic full | Weekly | 8 weekly | Medium-term recovery |
| Monthly backup copy | Monthly | 12 monthly | Longer-term retention |
| Yearly archive | Yearly if required by policy | As required by retention/regulation | Long-term retention |
| Critical VM replication to DR | Target ≤5 minutes where workload/WAN permits | Replica restore points as designed | Rapid site failover |
| Immutable retention | Minimum 14–30 days proposed | Final value per threat model and storage capacity | Ransomware protection |
| Configuration backup | Daily | 30 days minimum | Veeam configuration recovery |

5.4 Backup Security Controls

Do not use the production SAN as the only backup destination.

Place backup infrastructure in a dedicated backup/security VLAN with tightly controlled firewall rules.

Use separate administrative credentials for backup infrastructure; avoid routine domain-admin dependency where feasible.

Use an immutable repository so protected backup files cannot be deleted during the configured immutability period.

Encrypt backup data in transit and at rest where supported.

Restrict interactive logon to repositories and management servers.

Send backup alerts and security events to the monitoring/SIEM platform.

Regularly test restore of VMs, files and application data; a successful backup job alone is not sufficient proof of recoverability.

Maintain an exported/documented recovery runbook and protected copy of configuration information.

5.5 Veeam Components – High-Level

| Software / Role | Qty | Purpose |
| --- | --- | --- |
| Veeam Backup & Replication | 1 deployment | Central backup, restore, replication and backup-copy management. |
| Veeam licenses | As required by protected workloads | License quantity depends on final Veeam licensing model and protected workload count. |
| Veeam Backup Server | 1 VM on Primary Hyper-V cluster | Central backup/replication management; does not store the protected backup data itself |
| Hyper-V backup proxy capability | As sized | On-host or off-host data movement; dedicated proxy may be used for scale/performance. |
| Primary backup repository | 1 physical system | Separate physical repository for fast local restores; not part of Hyper-V cluster or production SAN |
| Immutable repository | 1 physical / supported immutable target | Separate hardened immutable repository for ransomware-resistant restore points |
| DR backup repository | 1 physical system | Independent physical off-site repository at DR for backup copies and recovery |
| Optional object storage capacity | As required | Additional immutable/off-site retention tier if approved. |

6. Virtual Workload and Software Architecture

The exact application VM inventory must be confirmed by the personalization/application supplier. The latest architecture places the infrastructure and application services listed below as VMs on the Hyper-V cluster, including API/integration, reverse proxy, monitoring, management tools and the Veeam Backup Server. Physical backup repositories remain outside the cluster. The following is a logical planning model, not a mandatory VM count.

| Logical Workload | Primary | DR | HA / Protection |
| --- | --- | --- | --- |
| Domain Controller / DNS 01 | VM | VM or independent instance | Native AD redundancy + Veeam backup |
| Domain Controller / DNS 02 | VM | Optional / recommended | Native AD redundancy + Veeam backup |
| Application Server 01 | VM | Replica | Clustered VM + Veeam replication |
| Application Server 02 / Workflow | VM | Replica | Application/load-balancing design as supported |
| Database Server(s) | VM(s) | Replica / DB-native DR where required | DB HA method to be confirmed with DB vendor |
| Integration / API | VM on Primary Hyper-V cluster | Replica VM at DR | Connected to DMZ/internal integration zone according to firewall policy |
| Monitoring Server | VM on Primary Hyper-V cluster | VM / backup at DR | Central monitoring; protected by cluster HA and backup |
| Veeam Backup Server | VM on Primary Hyper-V cluster | VM / recoverable at DR | Backup management/control plane only; repositories remain physical and separate |
| Security / Log integration | VM or external platform | As required | SIEM/EDR integration |
| Vendor utility / licensing services | As required | As required | Vendor-defined |
| Reverse Proxy / Web Server | VM on Primary Hyper-V cluster | Replica VM at DR | DMZ VLAN 100; firewall-isolated from application/database networks |
| Management Tools Server | VM on Primary Hyper-V cluster | VM / backup at DR | Infrastructure administration tools on Management & Monitoring VLAN |

6.1 Microsoft Software

Windows Server 2025 Datacenter (recommended for virtualization rights and cluster hosts), licensed by physical cores in accordance with Microsoft licensing rules.

Windows Server CALs for users/devices where required.

Microsoft SQL Server edition/licensing only if required by the application vendor; edition and HA licensing must be confirmed separately.

Windows Server Failover Clustering and Hyper-V roles.

Optional Windows Admin Center / System Center tooling depending on the final operational model.

7. High-Level Bill of Materials (BOM)

The BOM below defines the infrastructure classes and preliminary quantities. OEM model numbers, exact CPU SKUs, disk counts and license quantities should be selected only after final sizing and compatibility validation.

7.1 Primary Site – Hardware

| # | Item | Qty | High-Level Specification |
| --- | --- | --- | --- |
| 1 | Enterprise Hyper-V cluster server | 2 | 2U, dual CPU, ≥256 GB RAM, RAID1 boot SSD, 10/25GbE, dual storage paths, OOB management, dual PSU |
| 2 | Enterprise shared SAN | 1 | Dual-controller, SSD/all-flash preferred, multipath, snapshots, encryption; preliminary 10–15 TB usable |
| 3 | Core / distribution L3 switch | 2 | Redundant/stackable or MLAG-capable, 10/25Gb uplinks, VLAN/LACP/ACL/QoS, redundant PSU |
| 4 | Managed access switch | 1 | 24-port managed PoE+, VLANs, LACP, SNMP; add second if access HA required |
| 5 | NGFW | 2 / HA pair preferred | IPS, application control, VPN, segmentation, logging; sizing based on encrypted throughput |
| 6 | Backup repository server/storage | 1 | Separate physical server/storage, independent from Hyper-V cluster and production SAN; sized from protected capacity, change rate and retention |
| 7 | Immutable backup repository | 1 | Separate physical hardened Linux repository or supported immutable appliance/object target; isolated from production failure domain |
| 8 | SFP+/SFP28/DAC/fiber modules | As required | OEM-supported optics/cables for redundant 10/25Gb paths |
| 9 | Rack PDUs | 2 | A/B redundant power feeds |
| 10 | Rack/cabling/patching | 1 lot | Rack, fiber/copper patching, labels and cable management |

7.2 DR Site – Hardware

| # | Item | Qty | High-Level Specification |
| --- | --- | --- | --- |
| 1 | Enterprise Hyper-V DR server | 2 | Compatible enterprise servers; capacity for Tier-1/Tier-2 recovery workloads |
| 2 | DR shared SAN | 1 | Dual-controller, SSD/all-flash preferred, sized for replicas and recovery space |
| 3 | DR core L3 switch | 2 | Redundant switching with 10/25Gb server/storage connectivity |
| 4 | DR access/management switch | 1 | Managed 24-port baseline; redundancy as required |
| 5 | DR NGFW | 2 / HA pair preferred | Secure WAN/VPN, segmentation and logging |
| 6 | DR backup repository | 1 | Separate physical off-site repository at DR; receives encrypted backup copies and supports DR recovery/testing |
| 7 | WAN termination / SD-WAN | As required | Encrypted inter-site connectivity sized for replication RPO |
| 8 | Rack/PDUs/cabling | 1 lot | Redundant power and structured cabling |

7.3 Software and Licensing

| # | Software | Qty / Basis | Purpose |
| --- | --- | --- | --- |
| 1 | Windows Server Datacenter | Per physical cores on all Hyper-V hosts | Hyper-V, WSFC and Windows Server VM virtualization rights |
| 2 | Windows Server CALs | Per user/device as applicable | Windows Server access licensing |
| 3 | Veeam Backup & Replication / Veeam Data Platform edition | Per final protected-workload licensing basis | Backup, recovery, replication, immutable/off-site copy workflows |
| 4 | Linux OS for hardened repository | 1–2 instances | Immutable Veeam repository; choose supported distribution/version |
| 5 | Database platform license | As application requires | Application database and HA/DR; exact edition TBC |
| 6 | NGFW subscriptions | Per firewall / term | IPS, malware protection, URL/app controls, support |
| 7 | Server/SAN/switch management/support | Per OEM / 3–5 years | Firmware, warranty, hardware replacement and support |
| 8 | EDR | Per server/workstation | Endpoint protection required by security architecture |
| 9 | Monitoring/SIEM integration | As required | Central monitoring, logging, alerting and audit |
| 10 | Backup repository OS / appliance licensing | As selected | Repository platform and support |

8. Sizing and Final Design Inputs Required

Before issuing the final priced BOM, the following inputs should be obtained from the application/personalization vendor and customer. These values materially affect server, SAN, WAN and backup sizing.

| Sizing Input | Why It Is Required |
| --- | --- |
| Final VM list and OS per VM | Determines compute, licensing and protection scope. |
| vCPU/RAM requirement per VM | Determines host CPU/RAM and N+1 failover capacity. |
| Database engine, edition and size | Determines DB licensing, HA design and storage performance. |
| Database IOPS / latency requirement | Determines SAN media, RAID and controller sizing. |
| Current data volume and 3–5 year growth | Determines production and backup capacity. |
| Daily data-change rate | Determines backup window, repository capacity and WAN replication bandwidth. |
| Required RPO per application | Determines replication frequency and technology. |
| Required RTO per service | Determines DR capacity and recovery automation. |
| Backup retention policy | Determines repository capacity and archive tier. |
| WAN bandwidth/latency between sites | Determines feasibility of ≤5 minute replication. |
| Number of users/operators/endpoints | Determines access switching, CALs and user network sizing. |
| Application clustering support | Determines whether HA is VM-level, application-level, DB-level or combined. |

8.1 N+1 Compute Rule

The two-node production cluster should be sized so that either node can run all Tier-1 workloads and the required subset of Tier-2 workloads during a node outage. Normal operation should therefore avoid sustained utilization levels that leave no failover headroom.

9. Implementation Approach

Confirm requirements, VM inventory, dependencies, RPO/RTO and capacity growth.

Finalize low-level design, IP plan, VLAN plan, firewall matrix, rack elevations and cable schedule.

Install racks, redundant power, switches, firewalls, servers and storage.

Configure SAN pools/LUNs, MPIO and isolated storage networks.

Install Windows Server, Hyper-V and WSFC; run full cluster validation.

Deploy workloads and validate application/database HA behavior.

Build DR cluster and establish secure inter-site connectivity.

Deploy Veeam backup infrastructure, immutable repository and DR backup copy/replication.

Execute backup/restore, node-failure, path-failure, switch-failure and DR tests.

Complete documentation, training, handover and acceptance.

9.1 Acceptance Tests

| Test | Acceptance Objective |
| --- | --- |
| Cluster node failure | Critical VMs restart successfully on surviving node. |
| Live migration | VMs move between nodes without unacceptable service impact. |
| Core link/switch failure | Connectivity remains available through redundant path. |
| SAN path/controller failure | CSV/LUN remains available through MPIO. |
| Backup job | Scheduled backup completes and is monitored. |
| Immutable backup test | Protected restore point cannot be prematurely deleted using normal administrative workflow. |
| VM restore | Full VM restore / recovery is demonstrated. |
| File/application restore | Granular restore is demonstrated for supported workloads. |
| DR replication | Critical VM replicas meet approved RPO. |
| DR failover drill | Services recover in documented dependency order and achieved RTO/RPO are recorded. |

10. Design Assumptions, Risks and Recommendations

| Area | Recommendation / Risk |
| --- | --- |
| Single core switch | Do not use one core switch for a mission-critical solution; use two. |
| Two-node cluster capacity | Each node must retain sufficient capacity for failover; otherwise HA exists logically but not operationally. |
| Shared SAN | Dual-controller SAN is locally resilient but remains a site-level dependency; DR replication is still required. |
| Backups on production storage | Not acceptable as the only backup copy. Use independent and immutable storage. |
| DR bandwidth | ≤5 minute synchronization cannot be guaranteed until change rate, WAN bandwidth and latency are measured. |
| Database HA | Do not assume VM failover alone satisfies database HA/RPO; confirm DB-native requirements with the application vendor. |
| RTO | RFP target must be validated by end-to-end application testing, not infrastructure boot time alone. |
| Security | Use MFA/PAM, EDR, SIEM integration, hardened OS baselines, restricted backup administration and NGFW segmentation. |
| Documentation | Maintain as-built diagrams, IP/VLAN/firewall matrices, backup/restore procedures and DR runbooks. |

11. Compliance Mapping Summary

| RFP Area | Solution Alignment |
| --- | --- |
| High availability | Two-node WSFC/Hyper-V clusters, redundant network and SAN paths. |
| DR architecture | Secondary two-node cluster, DR SAN, secure WAN and recovery runbook. |
| Backup | Automated Veeam backup for VMs/applications with centralized management. |
| Off-site copy | Backup copy to DR site. |
| Immutable storage | Veeam hardened repository or supported immutable target. |
| Ransomware recovery | Separated backup credentials, immutable copies and tested recovery. |
| Network segmentation | Dedicated VLANs/zones with firewall isolation. |
| Encryption | Encrypted communications; storage/backup encryption where supported. |
| Monitoring | Infrastructure and backup monitoring with SIEM integration. |
| Testing | Cluster, restore, replication and DR acceptance testing plus recurring drills. |

11.1 Physical vs. Virtual Component Summary

Virtual on Hyper-V clusters: DC/DNS, application, database, API/integration, reverse proxy, monitoring, management tools, Veeam Backup Server and other application VMs. Physical/separate infrastructure: Hyper-V hosts, SAN, core/access switches, firewalls, primary backup repository, immutable backup repository and DR backup repository.

12. Conclusion

The proposed architecture provides a practical enterprise platform for the RFP: local high availability through a two-node Hyper-V failover cluster, shared resilient SAN storage, redundant networking, security segmentation, a physically/logically separate DR environment and a ransomware-resilient backup design. Veeam is suitable as the backup and replication platform for this architecture, provided the final selected version is validated against the exact Hyper-V, Windows Server, storage and application versions at implementation.

The next engineering step should be a detailed sizing exercise followed by an OEM-specific BOM. At that stage, server CPU/RAM, SAN disk counts, switch port counts, firewall throughput, WAN bandwidth and Veeam repository capacity can be converted from preliminary allowances into tender-ready specifications.

Appendix A – Source Basis

RFP basis:

Digital Malawi Acceleration Project – RFP No. MW-PPPC-546386-GO-RFB, Section VII Purchaser’s Requirements.

Architectural requirements: multi-tier software, HA clustering, backup replication, DR synchronization and enterprise-grade redundant hardware.

Backup/DR requirements: automated backups, encryption, validation, off-site replication, immutable storage, centralized management, RTO/RPO, DR synchronization and testing.

Performance requirements: automatic failover initiation ≤60 seconds; full service recovery 5–15 minutes depending on architecture; primary-to-DR synchronization near-real-time or ≤5 minutes.

External product validation:

Veeam Backup & Replication User Guide – Microsoft Hyper-V platform support (accessed 26 Sep 2026).

Veeam Backup & Replication User Guide – Immutability for Backup Files (accessed 26 Sep 2026).

Veeam Backup & Replication User Guide – Hyper-V backup proxy/system requirements and replication prerequisites (accessed 26 Sep 2026).

Microsoft Learn – Failover clustering hardware requirements and storage options (accessed 26 Sep 2026).